The parameter values used in this study were selected as reasonable engineering estimates to represent the dynamic behaviour of the ankle joint, subtalar joint, toe joint, and longitudinal arch. The values were chosen to obtain a stable and physically realistic model. MATLAB simulations were then used to verify the dynamic response of the system. The healthy foot model was used as the baseline, and different foot conditions were obtained by modifying the parameters associated with the affected anatomical structure.

Table 1.0: Comparison of Model Parameters for Different Foot Conditions

| Parameter | Healthy | Flat Foot | High Arch | Weak Plantar Fascia | Ankle Instability | Rigid Foot(athelete) |
| --- | --- | --- | --- | --- | --- | --- |
| (J_a) | 0.0183 | Same | Same | Same | Same | Same |
| (J_s) | 0.008 | Same | Same | Same | Same | Same |
| (J_t) | 0.002 | Same | Same | Same | Same | Same |
| (m_h) | 0.08 | Same | Same | Same | Same | Same |
| (b_a) | 1.0 | Same | Same | Same | 0.5 ↓ | Same |
| (b_s) | 0.45 | Same | Same | Same | Same | Same |
| (b_t) | 0.18 | Same | Same | Same | Same | Same |
| (b_h) | 27 | 20 ↓ | 35 ↑ | 25 ↓ | Same | 32 ↑ |
| (k_a) | 170 | Same | Same | Same | 85 ↓ | Same |
| (k_s) | 70 | Same | Same | Same | Same | Same |
| (k_t) | 40 | Same | Same | Same | Same | Same |
| (k_h) | 25000 | 12500 ↓ | 50000 ↑ | 15000 ↓ | Same | 35000 ↑ |
| (k_{as}) | 10 | Same | Same | Same | Same | Same |
| (k_{ah}) | 15 | 8 ↓ | 25 ↑ | Same | Same | 20 ↑ |
| (k_{th}) | 12 | 6 ↓ | 20 ↑ | 4 ↓ | Same | 18 ↑ |

To ensure a fair comparison between different foot conditions, the same unit input was applied to all models. Therefore, any variation in the output responses is solely due to changes in the biomechanical parameters of the foot model.

U=    1

1

1

1

where the four inputs correspond to ankle torque ,subtalar torque ,toe torque , and ground reaction force.

OUTPUT VALUES BASED ON PARAMETERS—(RANGE)

Table 1.1: Estimated Output Ranges for Different Foot Conditions

| Condition | ( \theta_a ) (rad) | ( \theta_s ) (rad) | ( \theta_t ) (rad) | ( h ) (m) |
| --- | --- | --- | --- | --- |
| Healthy | 0.004–0.008 | 0.010–0.018 | 0.020–0.035 | 0.00002–0.00008 |
| Flat Foot | 0.006–0.012 | 0.014–0.025 | 0.028–0.050 | 0.00008–0.00020 |
| High Arch | 0.002–0.005 | 0.006–0.012 | 0.010–0.022 | 0.000005–0.00002 |
| Weak Plantar Fascia | 0.005–0.009 | 0.011–0.020 | 0.030–0.055 | 0.00005–0.00012 |
| Ankle Instability | 0.010–0.025 | 0.015–0.030 | 0.022–0.040 | 0.00003–0.00008 |
| Rigid Foot(athelete) | 0.003–0.006 | 0.008–0.015 | 0.015–0.028 | 0.00001–0.00004 |

These output ranges are engineering estimates based on the expected biomechanical behaviour of each foot condition relative to the healthy baseline model.

Plots represent the mean steady-state values derived from the simulated parameter ranges